Report of the Mini Project Done in Nest Digitals on

# HOME SAFETY DEVICE

Done by

NEXUS 4

### Ishan Jiji George

Manuel James

Sherry John

Sona Salu

### INTRODUCTION

Home safety has become a critical concern in modern households, necessitating smart and automated solutions to detect potential hazards effectively. This mini-project focuses on developing an IoT-based home safety system utilizing the MQ6 gas sensor, ESP8266 microcontroller, DHT11 temperature & humidity sensor, red LED, and buzzer to enhance security and hazard detection. The system continuously monitors air quality and environmental conditions, particularly gas leakage, and provides real-time alerts through visual and sound indicators. When hazardous gas levels exceed a defined threshold, the LED activates immediately, followed by the buzzer after 2 seconds, ensuring timely warnings. The ESP8266 enables cloud connectivity via Firebase, allowing remote monitoring and data logging. This implementation enhances home security, prevents accidents, and offers an accessible and scalable approach to smart home automation. The project emphasizes reliability, efficiency, and ease of integration into modern households.

### OBJECTIVE

The primary objective of this IoT-based home safety system is to enhance domestic security by providing real-time monitoring and hazard detection. The system utilizes an MQ6 gas sensor to detect harmful gas leaks, an ESP8266 microcontroller for wireless connectivity, a DHT11 sensor for environmental monitoring, and LED & buzzer indicators for immediate alerts. Upon detecting gas leakage, the LED turns on immediately, followed by the buzzer activation after 2 seconds, ensuring timely warnings for occupants. Data is logged in Firebase, enabling remote access and notifications for proactive safety measures. The project aims to create an efficient, scalable, and cost-effective solution for preventing potential accidents due to gas leaks and maintaining a secure living environment.

| COMPONENTS | REQUIREMENT |
| --- | --- |
| Esp8266 | 1 |
| MQ6 | 1 |
| DHT11 | 1 |
| Buzzer | 1 |
| Red Led | 1 |

COMPONENT REQUIRED

### WORKING PRINCIPLE

The IoT-based home safety system operates by continuously monitoring environmental conditions to detect gas leakage and trigger appropriate safety alerts. The MQ6 gas sensor detects harmful gases such as LPG and butane by measuring fluctuations in resistance based on gas concentration. When the detected gas level exceeds a predefined threshold, the ESP8266 microcontroller processes this data and activates a red LED immediately as a warning indicator. After 2 seconds, the buzzer turns on to ensure audible alerts for occupants. Additionally, the DHT11 sensor records temperature and humidity, providing comprehensive environmental monitoring. The ESP8266 transmits data to Firebase, where gas levels, temperature, humidity, and alert statuses are stored for remote access and monitoring. This enables users to receive notifications and take preventive actions efficiently, making the system an effective solution for home safety and hazard prevention.

### WORKING OF EACH COMPONENT IN THE HOME SAFETY SYSTEM

#### MQ6 Gas Sensor (Gas Detection)

- The MQ6 sensor detects gases like LPG, butane, methane, and other combustible substances.

- It consists of a sensing element that changes resistance based on the presence of gas molecules.

- The analog output varies with gas concentration, and the ESP8266 reads this signal to determine gas presence.

- When gas concentration exceeds a predefined threshold (e.g., 500 ADC value), it triggers alerts.

#### ESP8266 Microcontroller (Data Processing & Cloud Connectivity)

- The ESP8266 acts as the brain of the system, reading sensor values and controlling the output devices.

- It connects to WiFi and transmits data to Firebase, enabling remote monitoring.

- Based on sensor data, it activates the LED and buzzer when gas leakage is detected.

#### DHT11 Sensor (Environmental Monitoring)

- The DHT11 measures temperature and humidity to provide additional environmental insights.

- It operates using a capacitive humidity sensor and a thermistor to determine ambient conditions.

- ESP8266 logs the readings to Firebase, allowing real-time monitoring of room temperature and air quality.

#### Red LED Indicator (Visual Alert)

- The LED turns on immediately when MQ6 detects gas leakage.

- It acts as a visual warning, signaling occupants that harmful gas is present.

- Controlled by the ESP8266, it remains ON until the gas level returns to normal.

#### Buzzer (Audible Alert After 2 Seconds)

- After 2 seconds of LED activation, the buzzer turns on, ensuring a loud and noticeable alert.

- The delay ensures users notice the visual warning first, reducing panic while reinforcing awareness.

- The buzzer remains active until the gas concentration drops below the threshold.

#### 6. Firebase Realtime Database (Cloud Logging & Alerts)

- The ESP8266 uploads gas, temperature, and humidity levels to Firebase.

- Users can access this data remotely to ensure home safety.

- Firebase stores alert statuses (Gas Leak Detected or Safe), ensuring easy monitoring.

RESULT  OBTAINED

CODE FOR THE PROGRAM

#include <ESP8266WiFi.h>

#include <Firebase_ESP_Client.h>

#include <DHT.h>

#include <addons/TokenHelper.h>

#include <addons/RTDBHelper.h>

// WiFi Details

#define WIFI_SSID "OnePlus Nord CE4 Lite 5G"

#define WIFI_PASSWORD "MYpass@123"

// Firebase Details

#define API_KEY "AIzaSyCn697xTLvtC7Y2tQEvaTrWPwIB6bP3tjE"

#define DATABASE_URL "https://nexus4-39b9c-default-rtdb.firebaseio.com/"

#define USER_EMAIL "sonasalu2026@ec.ajce.in"

#define USER_PASSWORD "MYpassword@1234"

// Pins

#define DHTPIN 4

#define DHTTYPE DHT11

#define GAS_SENSOR 0

#define LED_PIN 14

#define BUZZER_PIN 12

DHT dht(DHTPIN, DHTTYPE);

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

unsigned long gasDetectionTime = 0;

bool gasDetected = false;

void setup()

{

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

dht.begin();

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected");

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

auth.user.email = USER_EMAIL;

auth.user.password = USER_PASSWORD;

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

}

void loop()

{

int gas = analogRead(GAS_SENSOR);

float temp = dht.readTemperature();

float hum = dht.readHumidity();

Serial.println("Gas: " + String(gas));

Serial.println("Temp: " + String(temp));

Serial.println("Humidity: " + String(hum));

// Send sensor data to Firebase

Firebase.RTDB.setInt(&fbdo, "/Sensor/Gas", gas);

Firebase.RTDB.setFloat(&fbdo, "/Sensor/Temperature", temp);

Firebase.RTDB.setFloat(&fbdo, "/Sensor/Humidity", hum);

if (gas > 120)

{

if (!gasDetected)

{

// First detection of gas

gasDetected = true;

gasDetectionTime = millis();

digitalWrite(LED_PIN, HIGH);  // Turn on LED immediately

Firebase.RTDB.setString(&fbdo, "/Alert", "Gas Detected");

}

// Check 2 seconds

if (millis() - gasDetectionTime >= 2000) {

digitalWrite(BUZZER_PIN, HIGH);  // Turn on buzzer after 2 seconds

}

}

else {

if (gasDetected)

{ gasDetected = false;

digitalWrite(LED_PIN, LOW);    // Turn off LED

digitalWrite(BUZZER_PIN, LOW); // Turn off buzzer

Firebase.RTDB.setString(&fbdo, "/Alert", "Safe");

}

}

delay(1000);

}

### Applications of the Home Safety System

- Gas Leakage Detection: Provides real-time monitoring for homes, kitchens, and industries to prevent hazardous gas leaks.

- Smart Home Automation: Integrates with IoT systems for automated alerts and responses.

- Industrial Safety: Used in factories handling combustible gases to enhance workplace safety.

- Health Protection: Prevents respiratory issues caused by exposure to toxic gases.

- Remote Monitoring: Enables users to track gas levels, temperature, and humidity from anywhere via Firebase cloud.

- Early Fire Hazard Warning: Detects potential fire risks due to gas leaks, preventing accidents.

- Home Security Enhancement: Works alongside other smart security devices for improved safety.

### ADVANTAGES OF THE SYSTEM

1.Real-Time Alerts: Immediate gas detection and warning through LED and buzzer.

2. Remote Accessibility: Users can monitor and receive notifications via Firebase from any location.

3.IoT Integration: Compatible with smart home automation for enhanced security.

 4.Cost-Effective & Scalable: Affordable sensors and microcontrollers make it easy to implement.

5.Automated Safety Measures: No manual intervention required; system responds autonomously.
6. Low Power Consumption: Energy-efficient components ensure minimal power usage.

7.User-Friendly Design: Simple setup and operation for homeowners and businesses.

### CONCLUSION

The IoT-based home safety system successfully demonstrates an efficient and automated approach to detecting gas leaks and monitoring environmental conditions. By integrating an MQ6 gas sensor, ESP8266 microcontroller, DHT11 temperature & humidity sensor, red LED, and buzzer, the system provides real-time alerts for potential hazards. The LED activates immediately upon gas detection, while the buzzer sounds after 2 seconds, ensuring timely warnings. Additionally, Firebase cloud integration enables remote monitoring, allowing users to stay informed about home safety status from anywhere.

This project enhances domestic security and serves as a cost-effective and scalable solution for preventing accidents due to gas leaks. It showcases the potential of IoT in smart home automation, emphasizing reliability, accessibility, and efficiency. Future improvements could include mobile notifications via Firebase Cloud Messaging (FCM) or automated ventilation control for proactive hazard mitigation.